SMART HELMET – WORK DIVISION

Team Split

Person       	                                                     Responsibility

---------                                                             ------------------------

Person-1 (karthik)                    →               Helmet hardware

Person-2 (padmavathi)           →               Helmet Arduino code

Person-3 (    -     )                     →                Bluetooth pairing

Person-4 (thrishitha)              →                Bike hardware (DC motor + relay)

Person-5 (    -     )                     →                Bike Arduino code

SMART HELMET WITH BIKE ENGINE CONTROL

BASIC IDEA

This project ensures that the bike (DC motor) runs only when the rider wears the helmet.

If the helmet is not worn, the bike engine remains OFF.

-->MAIN PARTS OF THE PROJECT

Helmet Unit

- pressure Sensor

- Arduino

- LED

- Buzzer

- Bluetooth module

Bike Unit

- Arduino

- Bluetooth module

- Relay module

- DC Motor with wheel (engine demo)

- External battery

FLOWCHART

START

↓

Power ON System

↓

Read IR Sensor

↓

Is Helmet Worn?

├── NO ──► Motor OFF

│           ↓

│        Continue

│

└── YES

↓

Blink LED once

↓

Beep Buzzer once

↓

Send Signal via Bluetooth

↓

Receive Signal on Bike Side

↓

Turn ON Relay

↓

DC Motor ON

↓

Continue Monitoring

BLOCK DIAGRAM

[ IR SENSOR ]

↓

[ HELMET ARDUINO ]

↓

[ LED + BUZZER ]   (One-time indication)

↓

[ BLUETOOTH TX ]

↓  ~ ~ ~ Wireless ~ ~ ~

[ BLUETOOTH RX ]

↓

[ BIKE ARDUINO ]

↓

[ RELAY MODULE ]

↓

[ DC MOTOR (ENGINE) ]

==================================================================================

PERSON-1: HELMET HARDWARE     (karthik)

Goal:

Helmet should detect wearing condition and send signal to Arduino.

Components Used (Helmet Side):

- Arduino UNO / Nano

- presser sensor

- LED

- Buzzer

- Bluetooth module (HC-05 / HC-06)

- Breadboard

- Jumper wires (Male–Male & Female–Female)

- Battery / Power bank (later)

-->STEP 1: Hardware Setup (NO CODE YET)

What Person 1 does:

- Insert breadboard

- Do NOT connect battery

- Do NOT connect Bluetooth TX/RX yet

Basic Power Lines

- Arduino 5V → Breadboard +

- Arduino GND → Breadboard −

-->STEP 2: Connect Output Indicators

LED

- LED long leg → Arduino D6

- LED short leg → 220Ω resistor → GND

Buzzer

- Buzzer (+) → Arduino D7

- Buzzer (−) → GND

-->STEP 3: Helmet Wear Sensor (Simple Button Example)

- One side of button → D2

- Other side → GND

- When helmet is worn → button pressed → signal HIGH

Hardware Check (Person 1)

Before giving to coder:

- No loose wires

- Correct pin numbers written on paper

- Arduino NOT connected to laptop

==================================================================================

Person 2: ARDUINO CODE  (padmavathi)

Goal

Upload code that:

- Detects helmet wear

- Turns ON LED + Buzzer

- Sends signal to Bluetooth

WHEN TO UPLOAD CODE? (VERY IMPORTANT)

Correct Time to Upload Code

- After ALL hardware wiring is finished

- After Bluetooth TX/RX is DISCONNECTED

Wrong Time

- While wiring

- While Bluetooth TX/RX is connected

-->STEP 4: Preparing for Code Upload

Person 2 does:

Disconnect:

- Bluetooth TX

- Bluetooth RX

- Connect Arduino to Laptop via USB

- Open Arduino IDE

Select:

- Board: Arduino UNO / Nano

- Port: Correct COM port

-->STEP 5: Upload Code

Code Logic (Simple)

- Read button (helmet worn)

- If worn:

LED ON

Buzzer ON

Send 1 via Bluetooth

- Else:

LED OFF

Buzzer OFF

Send 0

Upload Process

- Click Upload

- Wait for “Done Uploading”

- Do NOT touch hardware during upload

-->STEP 6: After Upload

- Disconnect USB cable

- Now connect Bluetooth:

Bluetooth TX → Arduino RX

Bluetooth RX → Arduino TX

- Power Arduino using:

Battery / Power bank

PERSON-3: BLUETOOTH CONNECTION (Helmet ↔ Bike)

Goal

Helmet Bluetooth and Bike Bluetooth should pair correctly and send signal.

WHAT PERSON-3 DOES

-->Step-1: After Helmet Code Is Uploaded

- Helmet Arduino powered

- Bluetooth connected back (TX ↔ RX)

- No USB cable connected

-->Step-2: Power Both Bluetooth Modules

Helmet side Arduino → ON

Bike side Arduino → ON

Bluetooth LED Meaning

LED Pattern	    Meaning

----------                    -------

Fast blink	              Not paired

Slow blink	             Paired successfully

-->Step-3: Pair Bluetooth Modules

- Turn ON helmet Bluetooth

- Turn ON bike Bluetooth

- Wait 10–20 seconds

- Both LEDs blink slowly →  Paired

EG: Usually HC-05 password = 1234 or 0000

WHAT PERSON-3 SHOULD NOT DO

- Do not upload Arduino code

- Do not change wiring

- Do not connect USB during pairing

PERSON-3 SUCCESS CHECK

- Helmet worn → signal sent

- Bluetooth stays connected

- No disconnection when helmet moves

==================================================================================

PERSON-4: BIKE HARDWARE SETUP (DC MOTOR VERSION)  (Thrishitha)

Goal

- DC motor = bike engine

- Helmet worn → motor rotates

- Helmet not worn → motor stops

We use relay module because:

- Arduino cannot drive DC motor directly

- Relay safely switches motor power

COMPONENTS PERSON-4 USES

- Arduino UNO / Nano

- DC motor + tire

- Relay module (5V, single channel)

- Bluetooth module (ONLY place it, don’t connect TX/RX yet)

- External battery (9V / 12V / motor battery)

- Breadboard

- Jumper wires

IMPORTANT RULES FOR PERSON-4

Do NOT upload Arduino code

Do NOT connect Bluetooth TX/RX

Do NOT power motor directly from Arduino

Do NOT let motor wires touch Arduino pins

STEP-BY-STEP: BIKE HARDWARE WITH DC MOTOR

-->STEP-1: Basic Arduino Power

- Arduino 5V → Breadboard +

- Arduino GND → Breadboard −

This is ONLY for:

- Relay module

- Bluetooth module logic

-->STEP-2: Relay Module Connection (Control Side)

Relay Pin	        Connect To

--------                   ---------

VCC	                 Arduino 5V

GND	                 Arduino GND

IN	                 Arduino D8

->Relay IN pin is the control signal from Arduino.

-->STEP-3: DC Motor Connection (Power Side)

Relay has 3 screw terminals:

- COM (Common)

- NO (Normally Open)

- NC (Normally Closed)

- Use COM + NO

Wiring:

- Battery + → COM

- NO → DC Motor +

- DC Motor − → Battery −

->When relay turns ON → COM connects to NO → motor rotates

-->STEP-4: Bluetooth Placement (NO SIGNAL YET)

Place Bluetooth module on breadboard

Connect:

VCC → 5V

GND → GND

Do NOT connect TX/RX now

(Person-5 will do later)

PERSON-4 HARDWARE CHECKLIST

Before handing to Person-5:

- Relay clicks when powered

- Motor spins ONLY when relay is manually tested

- Motor powered from external battery

- No loose wires

- Arduino USB NOT connected

SAFE MANUAL TEST (OPTIONAL)

To confirm relay + motor:

- Briefly touch D8 to 5V

- Relay click

- Motor rotates

==================================================================================

PERSON-5: BIKE ARDUINO CODE UPLOAD

Goal

Bike Arduino should:

- Read Bluetooth signal

- Turn ON reElay only when helmet signal = YES

WHEN PERSON-5 UPLOADS CODE (IMPORTANT)

- Bike hardware completed

- Bluetooth TX & RX DISCONNECTED

- USB cable connected

❌ Never upload code with Bluetooth connected

WHAT PERSON-5 DOES (STEP-BY-STEP)

-->Step-1: Arduino IDE Setup

- Select correct board

- Select correct COM port

-->Step-2: Upload Bike Code

Logic:

- Receive '1' from helmet → relay ON

- No signal → relay OFF

-->Step-3: After Upload

- Disconnect USB

- Connect Bluetooth TX/RX

- Power Arduino using battery

WHAT PERSON-5 SHOULD NOT DO

- Change hardware wiring

- Touch relay connections during upload

PERSON-5 SUCCESS CHECK

Action	                      Result

-------                                 -------

Helmet worn	              Relay ON + LED ON

Helmet removed	     Relay OFF

==================================================================================

FULL PROJECT

Action	                             Result

------                                         ------

Helmet worn	                    Relay ON → Motor rotates

Helmet removed	         Relay OFF → Motor stops

Motor = bike engine demo