HARDWARE CONNECTION                                          BASIC SOFTWARE UPLOAD

Components Handles

- Arduino Nano

- Breadboard

- USB cable

- Pressure Sensor (FSR / switch)

- LED

- 220Ω resistor

- Buzzer

- HC-05 Bluetooth

- 10kΩ resistor

- Male-to-Male jumper wires

Place Arduino Nano

- Insert Arduino Nano in the center of breadboard

- USB port facing outside

- Connect USB cable to laptop

Power ON check:

- Arduino power LED should glow

Connect LED (Indicator)

LED purpose:

- Shows helmet status

- ON = Helmet worn

- OFF = Helmet not worn

Pin-to-Pin Connection:

| Arduino | LED |
| --- | --- |
| D6 | → 220Ω resistor |
| Resistor | → LED long leg (+) |
| LED short leg (–) | → GND |

Notes:

- Long leg = positive

- Short leg = negative

- Always use resistor to protect LED

Jumper wires:

- Male-to-Male

Connect Buzzer

Buzzer purpose:

- Gives alert sound

- Beeps when helmet not worn

Pin-to-Pin Connection:

| Arduino | Buzzer |
| --- | --- |
| D5 | → Buzzer + (red wire) |
| GND | → Buzzer – (black wire) |

Jumper wires:

- Male-to-Male

Connect Pressure Sensor (MOST IMPORTANT)

Pressure Sensor purpose:

- Detects whether helmet is worn

Components needed:

- Pressure Sensor

- 10kΩ resistor

Connections

| FSR Pin | Connects To |
| --- | --- |
| Pin 1 | 5V (Arduino) |
| Pin 2 | A0 (Arduino) |
| 10kΩ Resistor | Between A0 and GND |

Important:

- One side of resistor → A0

- Other side → GND

Arduino reads pressure from A0

Pressure Sensor Connection Logic

5V ---- FSR ---- A0 ---- 10kΩ ---- GND

Jumper wires:

- Male-to-Male

Connect Bluetooth HC-05

Bluetooth purpose:

- Sends helmet status to bike side

HC-05 Pin-to-Pin:

| HC-05 |  | Arduino Nano |
| --- | --- | --- |
| VCC | 5V |  |
| GND | GND |  |
| TXD | D2 |  |
| RXD | D3 (via voltage divider if possible) |  |

Note:

- RX & TX are crossed

- HC-05 LED should blink after power ON

Jumper wires:

- Male-to-Male

ARUDINO CODE – TESTING

#define LED_PIN 6

#define BUZZER_PIN 5

#define SWITCH_PIN 4

void setup() {

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(SWITCH_PIN, INPUT_PULLUP); // internal pull-up

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

}

void loop() {

// Read switch state

int switchState = digitalRead(SWITCH_PIN);

// Switch pressed (LOW because of pull-up)

if (switchState == LOW) {

// LED blink once

digitalWrite(LED_PIN, HIGH);

delay(300);

digitalWrite(LED_PIN, LOW);

// Buzzer beep once

digitalWrite(BUZZER_PIN, HIGH);

delay(300);

digitalWrite(BUZZER_PIN, LOW);

// Delay to avoid multiple triggers

delay(1000);

}

}