PERSON-4

This is for a Smart Bike / Smart Helmet type setup where:

- Arduino controls LED + relay (motor)

- Bluetooth sends ON/OFF command

- Switch is manual power

- 9V battery supplies power

Components used now:

- Arduino Nano

- Bluetooth (HC-05 / HC-06)

- Relay module

- DC Motor + Tyre

- LED

- 220Ω resistor

- 9V Battery

- Switch

- Jumper wires

STEP 1: POWER SUPPLY CONNECTION

9V Battery → Arduino Nano

Battery + (Red)  → Switch → VIN (Arduino Nano)

Battery – (Black) → GND (Arduino Nano)

Switch controls entire system power

STEP 2: Connect LED (Bike Indicator)

Purpose

- ON → Bike allowed

- OFF → Bike stopped

Pin-to-Pin Connection

Arduino D6 → 220Ω Resistor → LED long leg (+)

LED short leg (–) → GND

Notes

- Long leg = Positive

- Short leg = Negative

- Resistor is mandatory to protect LED

STEP 3: BLUETOOTH MODULE CONNECTION

Bluetooth VCC → 5V (Arduino)

Bluetooth GND → GND

Bluetooth TX → D2 (Arduino)

Bluetooth RX → D3 (Arduino)

TX ↔ RX cross connection
 Uses SoftwareSerial

Notes

- TX → RX (cross connection)

- Bluetooth LED should blink after power ON

Step-4 : Connect Relay Module

Purpose

Controls DC motor ON / OFF

Control Side Connections

Relay VCC → 5V (Arduino)

Relay GND → GND

Relay IN  → D8 (Arduino)

Step-5: Connect DC Motor Using Relay

Purpose

Rotate tyre only when relay is ON

Relay Output Connections

Battery + → COM (Relay)

NO → Motor +

Motor – → Battery –

Notes

- NO = Normally Open

- Motor runs only when relay is activated

Step-6: Fix Tyre to Motor

- Insert tyre on DC motor shaft

- Ensure it is tight

- No electrical connection needed

Step-8: Common Ground (VERY IMPORTANT)

All grounds must be connected together:

Arduino GND

Bluetooth GND

Relay GND

LED –

Battery –

WORKING FLOW  (Person-4)

- Switch ON → Arduino gets power

- Bluetooth connects

- Command received

- Arduino:

  - LED turns ON

  - Relay activates

  - Motor rotates → Bike moves

- Command OFF → Relay OFF → Motor stops

PIN SUMMARY (Easy View)

| Arduino Pin | Connected To |
| --- | --- |
| VIN | Battery + (via switch) |
| GND | Common Ground |
| D6 | LED (+ via 220Ω) |
| D2 | Bluetooth TX |
| D3 | Bluetooth RX |
| D8 | Relay IN |
| 5V | Bluetooth & Relay VCC |